Physics 112

Quiz #14

October 15, 1999

Name: _______________________

IF YOU WANT A QUESTION GRADED OUT OF THREE POINTS (–1 [MINUS ONE] FOR WRONG ANSWER!!) WRITE “3” IN SPACE PROVIDED ON EACH QUESTION.

e = 1.60 × 10–19 C

1. A proton is located at ((1 m, 0 m) and two protons are located at (+1 m, 0 m). A 1-C charge is located at the origin. What is the magnitude of the net electrical force experienced by the charge at the origin?

A.   0 N


B.   0.36 ( 10(9 N

C.   0.72 ( 10(9 N


D.   1.08 ( 10(9 N


E.   1.44 ( 10(9 N


F.   2.88 ( 10(9 N
G. 4.32 ( 10(9 N

2. A three-ohm resistor and a four-ohm resistor are connected in series to a battery. In a separate circuit, a three-ohm resistor and a four-ohm resistor are connected in parallel to a battery with the same battery voltage as in the first circuit. Which resistor will have the largest amount of current flowing through it? 

A. The three-ohm resistor in the series circuit. 

B. The four-ohm resistor in the series circuit.

C. The three-ohm resistor and the four-ohm resistor in the series circuit, which have the same amount of current flowing through them.

D. The three-ohm resistor in the parallel circuit.

E. The four-ohm resistor in the parallel circuit.

F. The three-ohm resistor and the four-ohm resistor in the parallel circuit, which have the same amount of current flowing through them.

Grade out of 3? Write “3” here: _____

3. Resistor A has twice the resistance of resistor B. They are connected in parallel to a battery. Then:

A. Resistor A dissipates four times as much power as resistor B.

B. Resistor A dissipates twice as much power as resistor B.

C. Resistor A dissipates the same amount of power as resistor B

D. Resistor A dissipates half as much power as resistor B.

E. Resistor A dissipates one fourth as much power as resistor B.

4. A 10-ohm and a 5-ohm resistor are connected in parallel to a battery as shown in the diagram. The power dissipated in the 5-ohm resistor is P5. What will happen to P5 if the 10-ohm resistor is removed, and another resistor with resistance less than 5 ohms is put in its place? (No other changes are made to the circuit.) 

Then P5: 

A. will increase.

B. will decrease.

C. will remain equal to 0 W.

D. will not change, but is not equal to 0 W.

E. might increase, decrease, or remain the same, depending on the precise value of the new resistance.

Grade out of 3? Write “3” here: _____

+





10 (





5 (





+





–





–








